| TECHNICAL BULLETIN LTB00369v2 25 MAR 2011 |  |
| --- | --- |

© Jaguar Land Rover Limited All rights reserved.. This reissue replaces all previous versions. Please destroy all previous versions. This bulletin supersedes TSB LTB00369/2011 dated 21 MAR 2011, which should either be destroyed or clearly marked to show it is no longer valid (e.g. with a line across the page). Only refer to the electronic version of this Technical Bulletin in TOPIx.

SECTION: 303-00 Vehicle Hesitation When Accelerating From Low Speed AFFECTED VEHICLE RANGE: Discovery 4 / LR4 (LA) Model Year:; 2010 Onwards. VIN:; 513326 Onwards.; Engine:; TDV6 2.7L Diesel TDV6 3.0L Diesel.. Range Rover Sport (LS) Model Year:; 2010 Onwards. VIN:; 215623 Onwards.; Engine:; TDV6 3.0L Diesel TDV8 3.6L Diesel.. MARKETS: All Except NAS CONDITION SUMMARY: Situation: Reduced engine performance with throttle demand at low vehicle speeds. This version has been issued for a change in the 'Section' number. Cause: Stop lamp switch incorrectly adjusted or brake booster free play. Suggested Customer Concern Code H20. Action: Should a customer express concern, follow the Service Instruction outlined below. WARRANTY: NOTE: Repair procedures are under constant review, and therefore times are subject to change; those quoted here must be taken as guidance only. Always refer to TOPIx to obtain the latest repair time. NOTE: DDW requires the use of causal part numbers. Labor only claims must show the causal part number with a quantity of zero. DESCRIPTION; SRO; TIME; CONDITION CODE; CAUSAL PART. Stoplamp switch - measure travel and adjust; 70.35.89.40; 0.2 hours; 07; XKB500110. NOTE: Normal Warranty procedures apply.

SERVICE INSTRUCTION: 1. Carry out the Brake Pedal Position (BPP) switch adjustment (see TOPIx Workshop Manual, 303-14 GENERAL PROCEDURES).. 2. Check for brake booster free play and tighten if required (see TOPIx Workshop Manual, 206-07 Power Brake Actuation)..